8D REPORT — 8D-ARGA-2024-012

Internal template TPL-8D-ARGA v2 (EN)

| Customer | Leitzaran Seating | Customer complaint no. | C-LEIZ-2024-0184 |
| --- | --- | --- | --- |
| Part number | AR-1007 | Description | Belt anchor flange (seat structure) |
| Characteristic | Hole diameter and burr (CC) | Complained lot | L24268-AR1007-01 |
| Date received | 2024-11-05 | Qty complained | 410 |
| Line / equipment | L1 — PR-250 / die MT-07 | Status | CLOSED |
| Closure date | 2025-01-21 | Version | v2 |

# D1 · Team

| Name | Role | Department |
| --- | --- | --- |
| Iñaki Etxeberria | Quality Manager (champion) | Quality |
| Maialen Irigoyen | Customer Quality Engineer (8D leader) | Quality |
| Unai Ezkurra | Toolroom Lead L1 | Toolroom |
| Koldo Arbizu | Maintenance Manager | Maintenance |
| Josu Elizalde | Afternoon Shift Leader L1/L3 | Production |
| Mikel Aranburu | Metrology & Lab Technician | Quality |
| Leire Sarasola | Supplier Quality Engineer | Purchasing |

Customer contact: A. Bengoetxea, Supplier Quality, Leitzaran Seating.

# D2 · Problem description (5W2H)

Problem statement

Leitzaran Seating reported 410 seat belt anchor flanges (AR-1007) with burr > 0.2 mm on the exit side of the anchor hole; the anchor bolt shoulder does not seat and the belt cannot be assembled. Lot L24268-AR1007-01.

| 5W2H |  |
| --- | --- |
| What | Burr 0.25–0.40 mm on the anchor hole (requirement ≤ 0.2 mm); hole Ø11.20–11.24 (spec Ø11.20 +0.15/0). CC characteristic. |
| Where | Leitzaran seat frame line, belt anchor bolting station. Origin: Arga L1, press PR-250, progressive die MT-07, piercing station P3. |
| When | Lot produced 24/09/2024 (afternoon shift), delivered 30/09/2024, detected 04/11/2024. |
| Who | Leitzaran assembly operator; notified by A. Bengoetxea. |
| Which | AR-1007 only (MT-07 is dedicated). |
| How | Rolled burr on the die side, consistent with a worn punch cutting edge. |
| How many | 410 NOK out of 2,572 delivered. |

Lot L24268-AR1007-01: produced 2024-09-24 (afternoon shift), press PR-250, 2,600 parts produced, 28 scrapped. Steel coil S-ULTZ-240223, e-coat chemistry S-ARAK-240141.

# D3 · Interim containment actions

Containment started on 2024-11-05. Scope: AR-1007 lots stamped with MT-07 from the complained lot up to the notification date.

| Lot | Part | Prod. date | Shift | Qty OK produced | Shipped (delivery notes) | Arga stock | Action | Result |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| L24268-AR1007-01 | AR-1007 | 2024-09-24 | afternoon | 2,572 | 2,572 (AL-24-00572) | 0 | Sorting at customer | 446 NOK |
| L24275-AR1007-01 | AR-1007 | 2024-10-01 | night | 1,690 | 1,690 (AL-24-00588) | 0 | Sorting at customer | 0 NOK |
| L24282-AR1007-01 | AR-1007 | 2024-10-08 | morning | 2,366 | 2,366 (AL-24-00603, AL-24-00604) | 0 | Sorting at customer | 0 NOK |
| L24289-AR1007-01 | AR-1007 | 2024-10-15 | afternoon | 1,984 | 1,984 (AL-24-00622) | 0 | Sorting at customer | 0 NOK |
| L24296-AR1007-01 | AR-1007 | 2024-10-22 | night | 2,360 | 2,360 (AL-24-00638) | 0 | Sorting at customer | 0 NOK |
| L24303-AR1007-01 | AR-1007 | 2024-10-29 | morning | 1,795 | 897 (AL-24-00654) | 898 | Blocked + 100 % sorting | 0 NOK |
| L24310-AR1007-01 | AR-1007 | 2024-11-05 | afternoon | 1,295 | 0 | 1,295 | Blocked + 100 % sorting | 0 NOK |

Arga stock (2,193 parts) blocked on 2024-11-05 and 100 % checked with burr gauge (0.2 mm) and plug gauge Ø11.20 go/no-go.

Leitzaran stock sorted by Arga on 2024-11-06–2024-11-07: 446 NOK, all from lot L24268-AR1007-01. Lots stamped after the punch sharpening of 28/09/2024 were 100 % OK.

NOK parts deburred? — Not allowed for CC characteristic; all NOK parts scrapped.

# D4 · Root cause analysis

Analysis on 2024-11-09 with toolroom and the die sharpening subcontractor S-ZIDA (Tratamientos Zidacos S.L.).

Ishikawa diagram (6M)

| Category | Potential cause | Verification / status |
| --- | --- | --- |
| Machine | Piercing punch P3 of MT-07 worn (cutting edge radius 0.18 mm, wear land 0.25 mm) | CONFIRMED |
| Method | Sharpening interval 40,000 strokes exceeded: counter at 47,500 strokes when the lot was stamped | CONFIRMED – die kept in production waiting for an S-ZIDA slot |
| Method | No spare punch set for MT-07 | CONFIRMED (contributing) |
| Material | Coil S-ULTZ-240223 (HSLA 500, 3.5 mm) | Ruled out – mechanical properties mid-range |
| Man | Die setting by afternoon shift | Ruled out – setup sheet complete |
| Measurement | Burr and hole checked at first and last part only | CONFIRMED (non-detection) |

5 Whys — occurrence

| Why |  |
| --- | --- |
| 1 | Why was there burr > 0.2 mm? — The cutting edge of punch P3 was worn and the effective clearance had increased. |
| 2 | Why was the punch worn? — It had reached 47,500 strokes, above the 40,000-stroke sharpening interval. |
| 3 | Why was the interval exceeded? — S-ZIDA had no slot for 10 days and the die was kept running to meet deliveries. |
| 4 | Why could it keep running? — There was no spare punch set and no interlock on the stroke counter of PR-250. |

5 Whys — non-detection

| Why |  |
| --- | --- |
| 1 | Why was the burr not detected? — Burr and hole diameter were checked only on the first and last part of the lot (PC-AR1007-01 v1). |
| 2 | Why? — Wear was considered slow and predictable; the control plan did not foresee a check during the run. |

Root cause — occurrence: Worn piercing punch P3 on die MT-07: sharpening overdue (above 40,000 strokes) because of subcontractor lead time and no spare punch.

Root cause — non-detection: Burr and hole diameter were only checked at first and last part; progressive wear during the run was not detected.

Root cause verification: S-ZIDA measurement of the removed punch (OT-MT-24-002): edge radius 0.18 mm. Trial on 2024-11-11 with the sharpened punch: burr 0.04–0.08 mm, Ø11.26 mm.

# D5 · Permanent corrective actions (chosen)

| Action | Type | Owner | Due date |
| --- | --- | --- | --- |
| Spare punch set for MT-07 (P3) held at Arga; die no longer waits for the subcontractor | Corrective – occurrence | Unai Ezkurra | 2024-11-26 |
| Stroke counter alarm on PR-250 at 38,000 strokes for MT-07 | Corrective – occurrence | Koldo Arbizu | 2024-11-19 |
| S-ZIDA contract: sharpening lead time ≤ 5 working days | Corrective – occurrence | Leire Sarasola | 2024-12-05 |
| Burr gauge + plug gauge check every 2 h during the run | Corrective – detection | Josu Elizalde | 2024-11-12 |

# D6 · Implementation and validation

Checks every 2 h in place since 2024-11-15; stroke alarm active since 2024-11-19; spare punch delivered on 2024-11-26. Operators of the three shifts trained on the new checks.

Validation results

Burr ≤ 0.10 mm in all 2-hourly checks for 8 weeks.

No further complaints from Leitzaran in the 60 days after implementation.

# D7 · Actions to prevent recurrence

| Action | Document / system | Owner | Due date | Status |
| --- | --- | --- | --- | --- |
| PFMEA: cause 'sharpening overdue', prevention = stroke alarm + spare punch | AMFE-AR1007-01 v2 | Unai Ezkurra | 2024-12-05 | Done |
| Control plan: 2-hourly burr and hole checks | PC-AR1007-01 v2 | Maialen Irigoyen | 2024-12-05 | Done |
| Die sharpening plan: lead time and spare punch requirement | PROC-L1-AFILADO-01 v2 | Koldo Arbizu | 2024-11-25 | Done |

# D8 · Closure and team recognition

8D closed on 2025-01-21. Leitzaran accepted the closure at the quarterly supplier review.

| Role | Name | Date | Signature |
| --- | --- | --- | --- |
| Customer Quality Engineer (8D leader) | Maialen Irigoyen | 2025-01-21 | signed |
| Quality Manager (champion) | Iñaki Etxeberria | 2025-01-21 | signed |