8D REPORT — OEM Norte Supplier Quality

Form OEMN-SQ-8D rev. 3 · to be returned within 10 working days

| Supplier | Componentes Arga S.L. | Supplier code | ARGA |
| --- | --- | --- | --- |
| OEM Norte complaint no. | C-OEMN-2025-0114 | Supplier 8D no. | 8D-ARGA-2025-002 |
| Part number | AR-1003 | Part description | Battery tray bracket LH |
| Special characteristic | MIG weld strength (CC) | Supplier lot | L25001-AR1003-01 |
| Complaint date | 2025-02-11 | Qty complained | 22 |
| Containment due (24 h) | 2025-02-12 | 8D due (10 WD) | 2025-02-25 |
| Report status | CLOSED | Closure date | 2025-04-30 |

# D1 · Team

| Name | Role | Department |
| --- | --- | --- |
| Iñaki Etxeberria | Quality Manager (champion) | Quality |
| Maialen Irigoyen | Customer Quality Engineer (8D leader) | Quality |
| Oihana Zabaleta | Welding Process Engineer | Engineering L2 |
| Xabier Urrutia | Production Manager L2 | Production |
| Mikel Aranburu | Metrology & Lab Technician | Quality |
| Koldo Arbizu | Maintenance Manager | Maintenance |

Customer contact: M. Ruiz, Supplier Quality Engineer, OEM Norte battery module assembly.

# D2 · Problem description (5W2H)

Problem statement

OEM Norte reported 22 battery tray brackets LH (AR-1003) broken at the MIG weld seam W2 during battery tray assembly (bolting and handling). Lack of fusion visible on the fracture surface. Lot L25001-AR1003-01.

| 5W2H |  |
| --- | --- |
| What | Lack of fusion along 15–30 mm of seam W2; fracture surface smooth with unfused areas. CC characteristic. |
| Where | OEM Norte battery tray assembly line. Origin: Arga L2, robot cell CR-01. |
| When | Lot produced 01/01/2025 (night shift, holiday with reduced crew), delivered 07/01/2025, detected 10/02/2025. |
| Who | OEM Norte assembly operators; notified by the OEM Norte SQE. |
| Which | AR-1003. AR-1004 lots of the same period sorted at the customer: no NOK. |
| How | Irregular bead, spatter on the part, porosity at bead start/stop; typical of an unstable arc. |
| How many | 22 NOK out of 1,679 delivered. |

Lot L25001-AR1003-01: produced 2025-01-01 (night shift), press PR-400, cell CR-01, 1,700 parts produced, 21 scrapped. Steel coil S-ULTZ-240204, MIG wire S-GOIE-240136, e-coat chemistry S-ARAK-240147.

# D3 · Interim containment actions

Containment started on 2025-02-11, within 24 h. Scope: all AR-1003 lots welded on CR-01 from the complained lot to the notification date.

| Lot | Part | Prod. date | Shift | Qty OK produced | Shipped (delivery notes) | Arga stock | Action | Result |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| L25001-AR1003-01 | AR-1003 | 2025-01-01 | night | 1,679 | 1,679 (AL-25-00013) | 0 | Sorting at customer | 27 NOK |
| L25008-AR1003-01 | AR-1003 | 2025-01-08 | afternoon | 2,463 | 2,463 (AL-25-00027, AL-25-00028) | 0 | Sorting at customer | 0 NOK |
| L25015-AR1003-01 | AR-1003 | 2025-01-15 | night | 2,984 | 2,984 (AL-25-00043) | 0 | Sorting at customer | 4 NOK |
| L25022-AR1003-01 | AR-1003 | 2025-01-22 | morning | 991 | 991 (AL-25-00057) | 0 | Sorting at customer | 0 NOK |
| L25029-AR1003-01 | AR-1003 | 2025-01-29 | afternoon | 1,673 | 1,673 (AL-25-00070) | 0 | Sorting at customer | 0 NOK |
| L25036-AR1003-01 | AR-1003 | 2025-02-05 | night | 796 | 398 (AL-25-00089) | 398 | Blocked + 100 % sorting | 2 NOK |

Arga stock (398 parts) blocked on 2025-02-11; 100 % visual inspection of seam W2 + pull-out test 5 parts per lot.

OEM Norte stock and parts in transit sorted by the Arga resident team on 2025-02-12–2025-02-14.

Rejects found in sorting were concentrated in night-shift lots; the holiday night shift of 01/01/2025 ran with 2 operators and no L2 technician.

Immediate action on CR-01 from 24/02/2025: gas nozzle and contact tip replaced every 8 h, recorded in the change log REG-L2-CR01-01.

# D4 · Root cause analysis

Root cause analysis on 2025-02-15–2025-02-17. Welding parameters were checked first: all within the WPS window (parameter lock and start-of-shift check in place).

Ishikawa diagram (6M)

| Category | Potential cause | Verification / status |
| --- | --- | --- |
| Machine | Contact tip worn (bore ovalised to 1.45 mm, nominal 1.20 mm) and gas nozzle ~40 % obstructed by spatter | CONFIRMED |
| Method | No defined frequency for nozzle / contact tip change ('when needed', operator judgement) | CONFIRMED |
| Man | Reduced crew on the holiday night shift of 01/01/2025 | CONFIRMED (contributing) |
| Method | Welding parameters of program P12 | Ruled out – within WPS-CR01-03, pendant locked |
| Material | MIG wire lot S-GOIE-240136, in use since 04/12/2024 | Ruled out – 3.1 certificate conform; no wire lot change during the affected lot |
| Material | Steel coil S-ULTZ-240204 (HSLA 420) | Ruled out |
| Measurement | Pull-out test 3 parts per lot passed (19.8 / 21.5 / 23.0 kN) | Sampling did not capture intermittent defect |
| Environment | Draught near CR-01 door (winter) | Ruled out – curtain closed |

5 Whys — occurrence

| Why |  |
| --- | --- |
| 1 | Why did seam W2 break? — Lack of fusion over part of the seam. |
| 2 | Why lack of fusion? — Unstable arc and poor gas shielding. |
| 3 | Why unstable arc? — Contact tip bore worn (1.45 mm) and nozzle obstructed by spatter, causing erratic current transfer and wire position. |
| 4 | Why were tip and nozzle worn? — No change frequency was defined; they were changed 'when needed' by operator judgement. |
| 5 | Why no frequency? — Consumable wear was not identified as a cause in the PFMEA or in IT-L2-CR01-03. |

5 Whys — non-detection

| Why |  |
| --- | --- |
| 1 | Why not detected at Arga? — The defect is intermittent; 3 pull-out tests per lot passed and visual inspection did not flag the irregular bead on the night shift. |

Root cause — occurrence: Worn contact tip and spatter-obstructed gas nozzle on CR-01, because no replacement frequency was defined for these consumables (changed by operator judgement).

Root cause — non-detection: Intermittent defect not captured by the 3-part pull-out sample per lot; no requirement to inspect consumables.

Root cause verification: Trial on 2025-02-17: 20 parts welded with the worn tip/nozzle removed from CR-01 → 5/20 with lack of fusion in macro-section; 20 parts with new tip/nozzle → 0/20. Tip wear measured after 8 h of production: bore 1.26–1.29 mm (acceptable).

# D5 · Permanent corrective actions (chosen)

| Action | Type | Owner | Due date |
| --- | --- | --- | --- |
| Change gas nozzle and contact tip every 8 h, recorded in REG-L2-CR01-01 | Corrective – occurrence | Xabier Urrutia | 2025-02-24 |
| Spare consumable kits stored at the cell; nozzle reamer cycle every 20 parts | Corrective – occurrence | Koldo Arbizu | 2025-02-25 |
| Update IT-L2-CR01-03 with the 8 h change and a visual check of the nozzle | Corrective – occurrence | Oihana Zabaleta | 2025-03-03 |
| Arc stability monitoring (weld data logger) — feasibility study | Corrective – detection | Oihana Zabaleta | 2025-04-12 |

# D6 · Implementation and validation

Change log REG-L2-CR01-01 started on 24/02/2025. IT-L2-CR01-03 v3 released on 03/03/2025 with the nozzle and contact tip change. Layered process audits on 12/03/2025 and 19/03/2025 (morning and afternoon shifts) confirmed compliance; the log was reviewed weekly by the L2 production manager for 4 weeks.

Validation results

Macro-sections weeks 10–17/2025: 16/16 with root penetration ≥ 1.2 mm.

Pull-out tests: minimum 20.9 kN, no value below 20 kN.

No complaints from OEM Norte on AR-1003/AR-1004 in the 60 days after implementation.

# D7 · Actions to prevent recurrence

| Action | Document / system | Owner | Due date | Status |
| --- | --- | --- | --- | --- |
| PFMEA: new cause 'contact tip / nozzle wear', prevention = change every 8 h, O 6→3 | AMFE-AR1003-01 v3 | Oihana Zabaleta | 2025-03-08 | Done |
| Control plan: consumable change frequency and log | PC-AR1003-01 v3 | Maialen Irigoyen | 2025-03-08 | Done |
| Work instruction: nozzle and contact tip change | IT-L2-CR01-03 v3 | Oihana Zabaleta | 2025-03-03 | Done |
| Read-across to CR-02: same consumable change frequency | IT-L2-CR02-02 v2 | Oihana Zabaleta | 2025-03-18 | Done |

# D8 · Closure and team recognition

8D closed on 2025-04-30; OEM Norte accepted the closure. The team presented the consumable change log at the plant quality meeting.

| Role | Name | Date | Signature |
| --- | --- | --- | --- |
| Customer Quality Engineer (8D leader) | Maialen Irigoyen | 2025-04-30 | signed |
| Quality Manager (champion) | Iñaki Etxeberria | 2025-04-30 | signed |
| OEM Norte SQE (acceptance) | M. Ruiz | 2025-05-03 | accepted |