8D REPORT — 8D-ARGA-2025-010

Internal template TPL-8D-ARGA v2 (EN)

| Customer | Leitzaran Seating | Customer complaint no. | C-LEIZ-2025-0170 |
| --- | --- | --- | --- |
| Part number | AR-1007 | Description | Belt anchor flange (seat structure) |
| Characteristic | Hole diameter and burr (CC) | Complained lot | L25196-AR1007-01 |
| Date received | 2025-08-26 | Qty complained | 520 |
| Line / equipment | L1 — PR-250 / die MT-07 | Status | CLOSED |
| Closure date | 2025-11-18 | Version | v2 |

# D1 · Team

| Name | Role | Department |
| --- | --- | --- |
| Iñaki Etxeberria | Quality Manager (champion) | Quality |
| Maialen Irigoyen | Customer Quality Engineer (8D leader) | Quality |
| Unai Ezkurra | Toolroom Lead L1 | Toolroom |
| Mikel Aranburu | Metrology & Lab Technician | Quality |
| Leire Sarasola | Supplier Quality Engineer | Purchasing |
| Josu Elizalde | Afternoon Shift Leader L1/L3 | Production |

Customer contact: A. Bengoetxea, Supplier Quality, Leitzaran Seating.

# D2 · Problem description (5W2H)

Problem statement

Leitzaran Seating reported 520 seat belt anchor flanges (AR-1007) with anchor hole diameter below tolerance and burr on the exit side; the anchor bolt does not enter. Lot L25196-AR1007-01.

| 5W2H |  |
| --- | --- |
| What | Hole Ø11.12–11.18 mm (spec Ø11.20 +0.15/0) and burr 0.22–0.34 mm (≤ 0.2 mm). CC characteristic. |
| Where | Leitzaran seat frame line. Origin: Arga L1, PR-250, progressive die MT-07, piercing station P3. |
| When | Lot produced 15/07/2025 (afternoon shift), delivered 19/07/2025, detected 25/08/2025. |
| Who | Leitzaran assembly operator. |
| Which | AR-1007 only. |
| How | Worn and micro-chipped punch edge; hole shrinks after springback in HSLA 500. |
| How many | 520 NOK out of 2,065 delivered. |

Lot L25196-AR1007-01: produced 2025-07-15 (afternoon shift), press PR-250, 2,100 parts produced, 35 scrapped. Steel coil S-ULTZ-250205, e-coat chemistry S-ARAK-250134.

# D3 · Interim containment actions

Containment started on 2025-08-26. Scope: AR-1007 lots stamped with MT-07 from the complained lot to the notification date.

| Lot | Part | Prod. date | Shift | Qty OK produced | Shipped (delivery notes) | Arga stock | Action | Result |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| L25196-AR1007-01 | AR-1007 | 2025-07-15 | afternoon | 2,065 | 2,065 (AL-25-00448) | 0 | Sorting at customer | 537 NOK |
| L25203-AR1007-01 | AR-1007 | 2025-07-22 | night | 1,285 | 1,285 (AL-25-00465) | 0 | Sorting at customer | 0 NOK |
| L25210-AR1007-01 | AR-1007 | 2025-07-29 | morning | 2,671 | 2,671 (AL-25-00479, AL-25-00480) | 0 | Sorting at customer | 0 NOK |
| L25217-AR1007-01 | AR-1007 | 2025-08-05 | afternoon | 2,969 | 2,969 (AL-25-00499, AL-25-00498) | 0 | Sorting at customer | 0 NOK |
| L25224-AR1007-01 | AR-1007 | 2025-08-12 | night | 1,983 | 1,983 (AL-25-00516) | 0 | Sorting at customer | 0 NOK |
| L25231-AR1007-01 | AR-1007 | 2025-08-19 | morning | 793 | 793 (AL-25-00531) | 0 | Sorting at customer | 0 NOK |
| L25238-AR1007-01 | AR-1007 | 2025-08-26 | afternoon | 2,570 | 0 | 2,570 | Blocked + 100 % sorting | 0 NOK |

Arga stock (2,570 parts) blocked on 2025-08-26 and 100 % checked with a new calibrated plug gauge and a dial burr gauge.

Leitzaran stock sorted on 2025-08-27–2025-08-28: 537 NOK, all from lot L25196-AR1007-01.

Lots stamped after the planned sharpening of 19/07/2025 (OT-MT-25-004) were 100 % conform.

# D4 · Root cause analysis

Analysis on 2025-08-30 with toolroom, S-ZIDA and the steel supplier S-ULTZ.

Ishikawa diagram (6M)

| Category | Potential cause | Verification / status |
| --- | --- | --- |
| Machine | Punch P3 cutting edge wear and micro-chipping | CONFIRMED – edge radius 0.20 mm, two chips of 0.1 mm |
| Method | Sharpening per plan PROC-L1-AFILADO-01 (60,000 strokes); counter at 58,000 strokes when the lot was stamped | Within plan – not retained as cause |
| Material | Coil S-ULTZ-250205 (HSLA 500) with tensile strength at the upper range (Rm 668 MPa, spec 560–700 MPa) | CONFIRMED (contributing) – accelerates punch wear |
| Measurement | Plug gauge Ø11.20 worn to Ø11.17 (calibration overdue by 2 months) | CONFIRMED (non-detection) |
| Man | 2-hourly checks performed and recorded | Ruled out – records complete |
| Environment | — | Not related |

5 Whys — occurrence

| Why |  |
| --- | --- |
| 1 | Why undersized hole and burr? — The punch cutting edge was worn and chipped. |
| 2 | Why was it worn and chipped? — High-strength coil at the upper end of the range increased edge loading near the end of the sharpening interval. |
| 3 | Why at the upper end? — HSLA 500 specification allows 560–700 MPa; no restriction for AR-1007. |
| 4 | Why did the punch chip? — Standard HSS punch without coating; not designed for Rm close to 700 MPa. |

5 Whys — non-detection

| Why |  |
| --- | --- |
| 1 | Why not detected at Arga? — The 2-hourly plug gauge check passed because the gauge itself was worn (Ø11.17). |
| 2 | Why was the gauge worn? — Calibration interval of 6 months not respected (overdue 2 months). |

Root cause — occurrence: Punch P3 edge wear and chipping, accelerated by a coil with tensile strength at the upper specification range.

Root cause — non-detection: Worn, out-of-calibration plug gauge accepted undersized holes during in-process checks.

Root cause verification: Trial on 2025-09-01: sharpened punch with coil S-ULTZ-250205 → Ø11.24–11.27, burr ≤ 0.08 mm. Worn plug gauge accepts Ø11.17 holes.

# D5 · Permanent corrective actions (chosen)

| Action | Type | Owner | Due date |
| --- | --- | --- | --- |
| Coated punch P3 (PM steel + TiCN) for MT-07 — order and trial | Corrective – occurrence | Unai Ezkurra | 2025-10-25 |
| HSLA 500 for AR-1007: Rm ≤ 650 MPa agreed with S-ULTZ | Corrective – occurrence | Leire Sarasola | 2025-09-25 |
| New plug gauge; calibration interval 3 months; gauge wear check weekly | Corrective – detection | Mikel Aranburu | 2025-09-02 |
| Burr measured with dial gauge at 4 positions every 2 h | Corrective – detection | Josu Elizalde | 2025-09-02 |

# D6 · Implementation and validation

New gauge in use since 2025-09-05; burr measurement at 4 positions since 2025-09-05. The sharpening interval of 60,000 strokes defined in PROC-L1-AFILADO-01 v3 is maintained; the coated punch is expected to extend punch life further.

Validation results

Hole Ø11.23–11.29 mm and burr ≤ 0.12 mm in all checks for 8 weeks.

No further complaints from Leitzaran in 60 days.

# D7 · Actions to prevent recurrence

| Action | Document / system | Owner | Due date | Status |
| --- | --- | --- | --- | --- |
| PFMEA: causes 'material at upper strength range' and 'gauge wear' | AMFE-AR1007-01 v3 | Unai Ezkurra | 2025-09-30 | Done |
| Control plan: burr at 4 positions, gauge calibration 3 months | PC-AR1007-01 v3 | Maialen Irigoyen | 2025-09-30 | Done |
| Coated punch trial on MT-07 | Toolroom project TR-25-07 | Unai Ezkurra | 2025-11-24 | Open – supplier delivery delayed |

# D8 · Closure and team recognition

8D closed on 2025-11-18 with the coated punch trial carried in the toolroom action plan. Leitzaran accepted the closure.

| Role | Name | Date | Signature |
| --- | --- | --- | --- |
| Customer Quality Engineer (8D leader) | Maialen Irigoyen | 2025-11-18 | signed |
| Quality Manager (champion) | Iñaki Etxeberria | 2025-11-18 | signed |