8D REPORT — OEM Norte Supplier Quality

Form OEMN-SQ-8D rev. 3 · to be returned within 10 working days

| Supplier | Componentes Arga S.L. | Supplier code | ARGA |
| --- | --- | --- | --- |
| OEM Norte complaint no. | C-OEMN-2025-0177 | Supplier 8D no. | 8D-ARGA-2025-011 |
| Part number | AR-1001 | Part description | Engine bay harness bracket |
| Special characteristic | Hole position (SC) | Supplier lot | L25216-AR1001-01 |
| Complaint date | 2025-09-16 | Qty complained | 180 |
| Containment due (24 h) | 2025-09-17 | 8D due (10 WD) | 2025-09-30 |
| Report status | CLOSED | Closure date | 2025-11-25 |

# D1 · Team

| Name | Role | Department |
| --- | --- | --- |
| Iñaki Etxeberria | Quality Manager (champion) | Quality |
| Maialen Irigoyen | Customer Quality Engineer (8D leader) | Quality |
| Unai Ezkurra | Toolroom Lead L1 | Toolroom |
| Koldo Arbizu | Maintenance Manager | Maintenance |
| Josu Elizalde | Afternoon Shift Leader L1/L3 | Production |

Customer contact: M. Ruiz, Supplier Quality Engineer, OEM Norte.

# D2 · Problem description (5W2H)

Problem statement

OEM Norte reported 180 engine bay harness brackets (AR-1001) with the two clip holes shifted 1.2 mm in X; the harness clip cannot be installed. Lot L25216-AR1001-01.

| 5W2H |  |
| --- | --- |
| What | Position of holes H1/H2 shifted +1.1 to +1.3 mm in X (tolerance ±0.3 mm, SC). |
| Where | OEM Norte front-end harness station. Origin: Arga L1, PR-250, progressive die MT-01. |
| When | Lot produced 04/08/2025 (afternoon shift), delivered 09 and 11/08/2025, detected 15/09/2025. |
| Who | OEM Norte assembly operator. |
| Which | Only parts stamped after a strip misfeed at the coil end. |
| How | Strip mis-registration: the pilot of station 2 was broken, so the strip was not re-centred. |
| How many | 180 NOK out of 2,586 delivered. |

Lot L25216-AR1001-01: produced 2025-08-04 (afternoon shift), press PR-250, 2,600 parts produced, 14 scrapped. Steel coil S-ULTZ-250133, e-coat chemistry S-ARAK-250136.

# D3 · Interim containment actions

Containment started on 2025-09-16, within 24 h. Scope: AR-1001 lots from the complained lot to the notification date.

| Lot | Part | Prod. date | Shift | Qty OK produced | Shipped (delivery notes) | Arga stock | Action | Result |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| L25216-AR1001-01 | AR-1001 | 2025-08-04 | afternoon | 2,586 | 2,586 (AL-25-00491, AL-25-00492) | 0 | Sorting at customer | 196 NOK |
| L25223-AR1001-01 | AR-1001 | 2025-08-11 | night | 893 | 893 (AL-25-00509, AL-25-00510) | 0 | Sorting at customer | 0 NOK |
| L25230-AR1001-01 | AR-1001 | 2025-08-18 | morning | 884 | 884 (AL-25-00525) | 0 | Sorting at customer | 0 NOK |
| L25237-AR1001-01 | AR-1001 | 2025-08-25 | afternoon | 1,985 | 1,985 (AL-25-00542) | 0 | Sorting at customer | 0 NOK |
| L25244-AR1001-01 | AR-1001 | 2025-09-01 | night | 1,177 | 1,177 (AL-25-00559) | 0 | Sorting at customer | 0 NOK |
| L25251-AR1001-01 | AR-1001 | 2025-09-08 | morning | 1,080 | 1,080 (AL-25-00574) | 0 | Sorting at customer | 0 NOK |
| L25258-AR1001-01 | AR-1001 | 2025-09-15 | afternoon | 1,965 | 0 | 1,965 | Blocked + 100 % sorting | 0 NOK |

Arga stock (1,965 parts) blocked on 2025-09-16 and 100 % checked on the checking fixture CF-AR1001.

OEM Norte stock sorted on 2025-09-17–2025-09-18.

# D4 · Root cause analysis

Analysis on 2025-09-20 with toolroom and maintenance.

Ishikawa diagram (6M)

| Category | Potential cause | Verification / status |
| --- | --- | --- |
| Machine | Pilot pin of station 2 of MT-01 broken after a misfeed at the coil end | CONFIRMED |
| Machine | Die protection sensor (misfeed) not connected after the last die change | CONFIRMED |
| Method | Die setup checklist without a check of the die protection sensors | CONFIRMED |
| Material | Coil S-ULTZ-250133 (DC04 1.5 mm): camber | Ruled out |
| Measurement | Checking fixture control once per shift | CONFIRMED (non-detection) |
| Man | — | Not related |

5 Whys — occurrence

| Why |  |
| --- | --- |
| 1 | Why were holes shifted? — The strip was not re-centred at station 2. |
| 2 | Why? — The pilot pin of station 2 was broken. |
| 3 | Why did it break? — A misfeed at the coil end was not stopped by the press. |
| 4 | Why was it not stopped? — The misfeed sensor cable was not connected after the last die change. |
| 5 | Why? — The die setup checklist (IT-L1-PR250-04 v1) did not require checking die protection sensors. |

5 Whys — non-detection

| Why |  |
| --- | --- |
| 1 | Why not detected at Arga? — Checking fixture control once per shift; the misfeed happened mid-shift. |

Root cause — occurrence: Broken pilot in MT-01 after an undetected misfeed, because the die protection sensor was not connected at die setup.

Root cause — non-detection: Checking fixture control frequency too low to detect a mid-shift event.

Root cause verification: Reproduced on 2025-09-21 with the pilot removed: holes shifted 1.1–1.3 mm.

# D5 · Permanent corrective actions (chosen)

| Action | Type | Owner | Due date |
| --- | --- | --- | --- |
| Replace pilot pins of MT-01; sensor connection with press interlock (no stroke without sensor signal) | Corrective – occurrence | Koldo Arbizu | 2025-09-26 |
| Die setup checklist: die protection sensors connected and tested | Corrective – occurrence | Unai Ezkurra | 2025-09-30 |
| Checking fixture control every 2 h and after every coil change | Corrective – detection | Josu Elizalde | 2025-09-23 |

# D6 · Implementation and validation

Implemented by 2025-09-30. IT-L1-PR250-04 updated to v2 with the sensor check.

Validation results

Hole position within ±0.15 mm in all checks during 8 weeks.

No recurrence at OEM Norte.

# D7 · Actions to prevent recurrence

| Action | Document / system | Owner | Due date | Status |
| --- | --- | --- | --- | --- |
| Read-across: sensor interlock on all progressive dies of PR-250 | PR-250 controller | Koldo Arbizu | 2025-10-31 | Done |
| Die setup instruction updated | IT-L1-PR250-04 v2 | Unai Ezkurra | 2025-09-30 | Done |

# D8 · Closure and team recognition

8D closed on 2025-11-25; accepted by OEM Norte.

| Role | Name | Date | Signature |
| --- | --- | --- | --- |
| Customer Quality Engineer (8D leader) | Maialen Irigoyen | 2025-11-25 | signed |
| Quality Manager (champion) | Iñaki Etxeberria | 2025-11-25 | signed |
| OEM Norte SQE (acceptance) | M. Ruiz | 2025-11-28 | accepted |