8D REPORT — OEM Norte Supplier Quality

Form OEMN-SQ-8D rev. 3 · to be returned within 10 working days

| Supplier | Componentes Arga S.L. | Supplier code | ARGA |
| --- | --- | --- | --- |
| OEM Norte complaint no. | C-OEMN-2025-0198 | Supplier 8D no. | 8D-ARGA-2025-014 |
| Part number | AR-1003 | Part description | Battery tray bracket LH |
| Special characteristic | MIG weld strength (CC) | Supplier lot | L25281-AR1003-01 |
| Complaint date | 2025-11-18 | Qty complained | 16 |
| Containment due (24 h) | 2025-11-19 | 8D due (10 WD) | 2025-12-02 |
| Report status | CLOSED | Closure date | 2026-01-27 |

# D1 · Team

| Name | Role | Department |
| --- | --- | --- |
| Iñaki Etxeberria | Quality Manager (champion) | Quality |
| Maialen Irigoyen | Customer Quality Engineer (8D leader) | Quality |
| Oihana Zabaleta | Welding Process Engineer | Engineering L2 |
| Xabier Urrutia | Production Manager L2 | Production |
| Ander Goñi | Night Shift Leader L2 | Production |
| Mikel Aranburu | Metrology & Lab Technician | Quality |

Customer contact: M. Ruiz, Supplier Quality Engineer, OEM Norte battery module assembly.

# D2 · Problem description (5W2H)

Problem statement

During its product audit (cross-sections), OEM Norte found 16 battery tray brackets LH (AR-1003) with a crack at the root of MIG weld seam W2. Lot L25281-AR1003-01. OEM Norte requested reference to previous 8D reports on the same part.

| 5W2H |  |
| --- | --- |
| What | Crack at the weld root of seam W2, length 3–8 mm, lack of fusion at the root. CC characteristic. |
| Where | OEM Norte product audit lab. Origin: Arga L2, robot cell CR-01. |
| When | Lot produced 08/10/2025 (night shift), delivered 11/10/2025, detected 17/11/2025. |
| Who | OEM Norte product audit team. |
| Which | AR-1003; AR-1004 (same cell) included in containment as read-across. |
| How | Macro-section: root penetration 0.3–0.7 mm; spatter inclusions at the root. |
| How many | 16 NOK out of 788 delivered. |

Lot L25281-AR1003-01: produced 2025-10-08 (night shift), press PR-400, cell CR-01, 800 parts produced, 12 scrapped. Steel coil S-ULTZ-250183, MIG wire S-GOIE-250130, e-coat chemistry S-ARAK-250141.

History: this part/defect was already addressed in 8D-ARGA-2025-002 (lack of fusion, lot L25001-AR1003-01), which introduced the change of gas nozzle and contact tip every 8 h. This report checks whether that action was effective.

# D3 · Interim containment actions

Containment started on 2025-11-18, within 24 h. Scope: AR-1003 and AR-1004 lots welded on CR-01 from the complained lot to the notification date.

| Lot | Part | Prod. date | Shift | Qty OK produced | Shipped (delivery notes) | Arga stock | Action | Result |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| L25281-AR1003-01 | AR-1003 | 2025-10-08 | night | 788 | 788 (AL-25-00646) | 0 | Sorting at customer | 19 NOK |
| L25288-AR1003-01 | AR-1003 | 2025-10-15 | night | 2,062 | 2,062 (AL-25-00662) | 0 | Sorting at customer | 4 NOK |
| L25295-AR1003-01 | AR-1003 | 2025-10-22 | morning | 1,581 | 1,581 (AL-25-00678) | 0 | Sorting at customer | 0 NOK |
| L25302-AR1003-01 | AR-1003 | 2025-10-29 | afternoon | 1,976 | 1,976 (AL-25-00695) | 0 | Sorting at customer | 0 NOK |
| L25309-AR1003-01 | AR-1003 | 2025-11-05 | night | 2,848 | 2,848 (AL-25-00711) | 0 | Sorting at customer | 2 NOK |
| L25316-AR1003-01 | AR-1003 | 2025-11-12 | morning | 1,767 | 883 (AL-25-00728) | 884 | Blocked + 100 % sorting | 0 NOK |
| L25282-AR1004-01 | AR-1004 | 2025-10-09 | afternoon | 2,179 | 2,179 (AL-25-00649, AL-25-00650) | 0 | Sorting at customer | 0 NOK |
| L25289-AR1004-01 | AR-1004 | 2025-10-16 | night | 2,457 | 2,457 (AL-25-00664, AL-25-00665) | 0 | Sorting at customer | 0 NOK |
| L25296-AR1004-01 | AR-1004 | 2025-10-23 | morning | 1,086 | 1,086 (AL-25-00680, AL-25-00681) | 0 | Sorting at customer | 0 NOK |
| L25303-AR1004-01 | AR-1004 | 2025-10-30 | afternoon | 2,082 | 2,082 (AL-25-00698, AL-25-00699) | 0 | Sorting at customer | 0 NOK |
| L25310-AR1004-01 | AR-1004 | 2025-11-06 | night | 1,184 | 1,184 (AL-25-00713) | 0 | Sorting at customer | 0 NOK |
| L25317-AR1004-01 | AR-1004 | 2025-11-13 | morning | 1,581 | 1,581 (AL-25-00731) | 0 | Sorting at customer | 0 NOK |

Arga stock (884 parts) blocked on 2025-11-18; 100 % visual of seam W2 + pull-out test 5 parts per lot + 1 macro-section per lot.

OEM Norte stock and parts in transit sorted by the Arga resident team on 2025-11-19–2025-11-21 (visual + dye penetrant).

All NOK parts found in sorting came from lots welded on the night shift.

Interim action from 21/11/2025: toolbox talk to the night shift of L2 and handwritten addendum at CR-01: nozzle and contact tip change at 22:00.

# D4 · Root cause analysis

Review of 8D-ARGA-2025-002 and of its actions on 2025-11-22. The change log REG-L2-CR01-01 was analysed for September and October 2025.

Ishikawa diagram (6M)

| Category | Potential cause | Verification / status |
| --- | --- | --- |
| Method | Nozzle/contact tip change not performed on the night shift: log REG-L2-CR01-01 shows the preventive change on 103 of 106 morning/afternoon shifts but only on 22 of 53 nights (Sep–Oct 2025) | CONFIRMED |
| Method | IT-L2-CR01-03 v3 §5.2 defines the change at the start of the morning (06:00) and afternoon (14:00) shifts only | CONFIRMED |
| Man | Night crew not trained on the 8 h rule; no shift-start meeting at 22:00 | CONFIRMED (contributing) |
| Machine | Contact tip in use up to 16 h at the end of the night shift (last change 14:00) | CONFIRMED |
| Material | MIG wire lot S-GOIE-250130, in use on CR-01 since 24/09/2025 | Ruled out – 3.1 certificate conform; program P12 unchanged at spool change |
| Material | Steel coil S-ULTZ-250183 (HSLA 420) | Ruled out |
| Measurement | Pull-out test 3 parts per lot passed (18.6 / 19.1 / 22.0 kN) — values close to the limit without reaction | CONFIRMED (non-detection) |
| Environment | — | Not related |

5 Whys — occurrence

| Why |  |
| --- | --- |
| 1 | Why did seam W2 crack at the root? — Lack of fusion at the root with spatter inclusions. |
| 2 | Why lack of fusion? — Unstable arc and poor gas shielding from a worn contact tip and an obstructed nozzle. |
| 3 | Why were they worn on 08/10/2025? — No change was made at the start of the night shift; the tip had been in use since 14:00. |
| 4 | Why no change on the night shift? — IT-L2-CR01-03 v3 lists the change only for the morning and afternoon shifts; the 8 h rule of 8D-ARGA-2025-002 was written into the instruction as 'start of shift' for the two day shifts. |
| 5 | Why was the gap not found earlier? — The effectiveness check of 8D-ARGA-2025-002 (layered audits of 12 and 19/03/2025) covered the morning and afternoon shifts only. |

5 Whys — non-detection

| Why |  |
| --- | --- |
| 1 | Why not detected at Arga? — Pull-out tests passed with values close to the 18 kN limit; the control plan has no reaction rule for results near the limit. |
| 2 | Why? — Only a lower specification limit was defined; no warning limit. |

Root cause — occurrence: The corrective action of 8D-ARGA-2025-002 (nozzle and contact tip change every 8 h) was not standardised for the night shift: IT-L2-CR01-03 v3 only defines it for the morning and afternoon shifts, so on the night shift the consumables stayed in use up to 16 h.

Root cause — non-detection: No warning limit or reaction rule for pull-out results close to the specification limit.

Root cause verification: Trial on 2025-11-24: tip after 16 h → bore 1.38–1.42 mm and 3/10 parts with root penetration < 1.0 mm; tip after 8 h → 1.25–1.28 mm and 10/10 conform.

# D5 · Permanent corrective actions (chosen)

| Action | Type | Owner | Due date |
| --- | --- | --- | --- |
| Include the 22:00 change (night shift) in IT-L2-CR01-03 and in the cell checklist | Corrective – occurrence | Oihana Zabaleta | 2026-01-30 |
| Night shift leader signs the change log every night; weekly review by L2 production manager | Corrective – occurrence | Ander Goñi | 2025-11-23 |
| Pull-out warning limit 20 kN: below it, 5 extra parts + macro-section | Corrective – detection | Mikel Aranburu | 2025-12-02 |
| Layered process audits on all three shifts (including night) for 3 months | Verification | Maialen Irigoyen | 2026-02-16 |

# D6 · Implementation and validation

Night-shift change applied from 21/11/2025 (toolbox talk + handwritten addendum at the cell). Warning limit for pull-out test in PC-AR1003-01 from 2025-12-02.

Validation results

Change log REG-L2-CR01-01, 21/11–31/12/2025: night-shift preventive change recorded on 28 of 35 nights.

Pull-out tests: minimum 20.2 kN, 2 warning-limit reactions, both conform after macro-section.

No further complaints from OEM Norte in the 60 days after implementation.

# D7 · Actions to prevent recurrence

| Action | Document / system | Owner | Due date | Status |
| --- | --- | --- | --- | --- |
| Work instruction: night-shift change at 22:00 | IT-L2-CR01-03 v4 | Oihana Zabaleta | 2026-01-30 | Open – draft v4 under review (action plan AP-2025-031) |
| PFMEA: detection rating of pull-out test reviewed (warning limit) | AMFE-AR1003-01 v4 | Oihana Zabaleta | 2025-12-18 | Done |
| Control plan: pull-out warning limit | PC-AR1003-01 v4 | Maialen Irigoyen | 2025-12-18 | Done |
| Lessons learned: verify 8D effectiveness on every shift | LL-2025-11 | Maialen Irigoyen | 2025-12-28 | Done |

Open D7 actions are tracked in the plant action plan (AP-2025-031) and reviewed at the monthly quality meeting.

# D8 · Closure and team recognition

8D closed on 2026-01-27 with the work instruction update carried in the action plan. OEM Norte accepted the closure.

| Role | Name | Date | Signature |
| --- | --- | --- | --- |
| Customer Quality Engineer (8D leader) | Maialen Irigoyen | 2026-01-27 | signed |
| Quality Manager (champion) | Iñaki Etxeberria | 2026-01-27 | signed |
| OEM Norte SQE (acceptance) | M. Ruiz | 2026-01-30 | accepted |