8D REPORT — 8D-ARGA-2026-001

Internal template TPL-8D-ARGA v2 (EN)

| Customer | Leitzaran Seating | Customer complaint no. | C-LEIZ-2026-0107 |
| --- | --- | --- | --- |
| Part number | AR-1011 | Description | Bracket for seat frame |
| Characteristic | Spot weld strength (CC) | Complained lot | L25363-AR1011-01 |
| Date received | 2026-02-10 | Qty complained | 35 |
| Line / equipment | L2 — SP-01 | Status | CLOSED |
| Closure date | 2026-04-21 | Version | v2 |

# D1 · Team

| Name | Role | Department |
| --- | --- | --- |
| Iñaki Etxeberria | Quality Manager (champion) | Quality |
| Maialen Irigoyen | Customer Quality Engineer (8D leader) | Quality |
| Oihana Zabaleta | Welding Process Engineer | Engineering L2 |
| Koldo Arbizu | Maintenance Manager | Maintenance |
| Xabier Urrutia | Production Manager L2 | Production |

Customer contact: A. Bengoetxea, Supplier Quality, Leitzaran Seating.

# D2 · Problem description (5W2H)

Problem statement

Leitzaran Seating reported 35 seat frame brackets (AR-1011) with a stuck (unfused) spot weld that separated during assembly. Lot L25363-AR1011-01.

| 5W2H |  |
| --- | --- |
| What | Spot weld S3 separated without nugget (stuck weld). CC characteristic. |
| Where | Leitzaran seat frame line. Origin: Arga L2, spot welding gun SP-01. |
| When | Lot produced 29/12/2025 (afternoon shift), delivered 02/01/2026, detected 09/02/2026. |
| Who | Leitzaran assembly operator. |
| Which | AR-1011. |
| How | Electrode cap face grown to 8.5 mm (nominal 6.0 mm) → current density too low. |
| How many | 35 NOK out of 1,080 delivered. |

Lot L25363-AR1011-01: produced 2025-12-29 (afternoon shift), press PR-250, cell SP-01, 1,100 parts produced, 20 scrapped. Steel coil S-ULTZ-250164, e-coat chemistry S-ARAK-250147.

# D3 · Interim containment actions

Containment started on 2026-02-10. Scope: AR-1011 lots from the complained lot until notification.

| Lot | Part | Prod. date | Shift | Qty OK produced | Shipped (delivery notes) | Arga stock | Action | Result |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| L25363-AR1011-01 | AR-1011 | 2025-12-29 | afternoon | 1,080 | 1,080 (AL-26-00004) | 0 | Sorting at customer | 41 NOK |
| L26005-AR1011-01 | AR-1011 | 2026-01-05 | night | 2,469 | 2,469 (AL-26-00019, AL-26-00020) | 0 | Sorting at customer | 0 NOK |
| L26012-AR1011-01 | AR-1011 | 2026-01-12 | morning | 2,862 | 2,862 (AL-26-00033) | 0 | Sorting at customer | 0 NOK |
| L26019-AR1011-01 | AR-1011 | 2026-01-19 | afternoon | 788 | 788 (AL-26-00049) | 0 | Sorting at customer | 0 NOK |
| L26026-AR1011-01 | AR-1011 | 2026-01-26 | night | 2,945 | 2,945 (AL-26-00063) | 0 | Sorting at customer | 0 NOK |
| L26033-AR1011-01 | AR-1011 | 2026-02-02 | morning | 2,876 | 2,876 (AL-26-00075) | 0 | Sorting at customer | 0 NOK |
| L26040-AR1011-01 | AR-1011 | 2026-02-09 | afternoon | 2,391 | 0 | 2,391 | Blocked + 100 % sorting | 0 NOK |

Arga stock (2,391 parts) blocked on 2026-02-10; chisel test 100 % on spot S3.

Leitzaran stock sorted on 2026-02-11–2026-02-12.

# D4 · Root cause analysis

Analysis on 2026-02-14.

Ishikawa diagram (6M)

| Category | Potential cause | Verification / status |
| --- | --- | --- |
| Machine | Tip dresser of SP-01 cutter broken; fault alarm bypassed during the holiday period | CONFIRMED |
| Method | No check of the electrode face diameter | CONFIRMED |
| Material | Coil S-ULTZ-250164 (HSLA 340) | Ruled out |
| Measurement | Chisel test only at start of shift | CONFIRMED (non-detection) |
| Man | Reduced crew during holiday period | Contributing |
| Environment | — | Not related |

5 Whys — occurrence

| Why |  |
| --- | --- |
| 1 | Why stuck weld? — Current density too low to form a nugget. |
| 2 | Why? — Electrode face grew to 8.5 mm. |
| 3 | Why? — Tip dressing did not take place: the dresser cutter was broken. |
| 4 | Why was welding not stopped? — The dresser fault alarm had been bypassed. |

5 Whys — non-detection

| Why |  |
| --- | --- |
| 1 | Why not detected? — Chisel test only at the start of each shift. |

Root cause — occurrence: Electrode caps not dressed because of a broken tip dresser whose alarm was bypassed.

Root cause — non-detection: Low chisel test frequency and no face diameter check.

Root cause verification: Trial on 2026-02-15: 8.5 mm face → 6/10 stuck welds; dressed 6.0 mm face → 10/10 nugget ≥ 5.2 mm.

# D5 · Permanent corrective actions (chosen)

| Action | Type | Owner | Due date |
| --- | --- | --- | --- |
| Tip dresser fault interlocked with welding (bypass requires maintenance key) | Corrective – occurrence | Koldo Arbizu | 2026-02-20 |
| Face diameter check with template every 2 h | Corrective – occurrence | Xabier Urrutia | 2026-02-17 |
| Chisel test 1 part every 2 h | Corrective – detection | Xabier Urrutia | 2026-02-17 |

# D6 · Implementation and validation

Implemented by 2026-02-24. IT-L2-SP01-02 updated to v5.

Validation results

All chisel tests conform for 8 weeks.

No recurrence at Leitzaran.

# D7 · Actions to prevent recurrence

| Action | Document / system | Owner | Due date | Status |
| --- | --- | --- | --- | --- |
| Bypass control of safety/quality alarms | Maintenance procedure | Koldo Arbizu | 2026-03-27 | Done |

# D8 · Closure and team recognition

8D closed on 2026-04-21; accepted by Leitzaran.

| Role | Name | Date | Signature |
| --- | --- | --- | --- |
| Customer Quality Engineer (8D leader) | Maialen Irigoyen | 2026-04-21 | signed |
| Quality Manager (champion) | Iñaki Etxeberria | 2026-04-21 | signed |