8D REPORT — OEM Norte Supplier Quality

Form OEMN-SQ-8D rev. 3 · to be returned within 10 working days

| Supplier | Componentes Arga S.L. | Supplier code | ARGA |
| --- | --- | --- | --- |
| OEM Norte complaint no. | C-OEMN-2026-0121 | Supplier 8D no. | 8D-ARGA-2026-003 |
| Part number | AR-1008 | Part description | Tailgate hinge reinforcement plate |
| Special characteristic | Hole position (SC) | Supplier lot | L26070-AR1008-01 |
| Complaint date | 2026-04-21 | Qty complained | 90 |
| Containment due (24 h) | 2026-04-22 | 8D due (10 WD) | 2026-05-05 |
| Report status | CLOSED | Closure date | 2026-06-28 |

# D1 · Team

| Name | Role | Department |
| --- | --- | --- |
| Iñaki Etxeberria | Quality Manager (champion) | Quality |
| Maialen Irigoyen | Customer Quality Engineer (8D leader) | Quality |
| Unai Ezkurra | Toolroom Lead L1 | Toolroom |
| Josu Elizalde | Afternoon Shift Leader L1/L3 | Production |
| Mikel Aranburu | Metrology & Lab Technician | Quality |

Customer contact: M. Ruiz, Supplier Quality Engineer, OEM Norte.

# D2 · Problem description (5W2H)

Problem statement

OEM Norte reported 90 tailgate hinge reinforcement plates (AR-1008) with hole position out of tolerance; the hinge does not align. Lot L26070-AR1008-01.

| 5W2H |  |
| --- | --- |
| What | Hole position shifted 0.6–0.9 mm in Y (tolerance ±0.3 mm, SC). |
| Where | OEM Norte body shop, tailgate line. Origin: Arga L1, PR-250, die MT-08. |
| When | Lot produced 11/03/2026 (night shift, die changed at 23:30), delivered 18/03/2026, detected 20/04/2026. |
| Who | OEM Norte body shop. |
| Which | AR-1008, parts stamped after the night-shift die change. |
| How | Die MT-08 positioned against the wrong stop block on the bolster. |
| How many | 90 NOK out of 983 delivered. |

Lot L26070-AR1008-01: produced 2026-03-11 (night shift), press PR-250, 1,000 parts produced, 17 scrapped. Steel coil S-ULTZ-260123, e-coat chemistry S-ARAK-260124.

# D3 · Interim containment actions

Containment started on 2026-04-21, within 24 h. Scope: AR-1008 lots from the complained lot until notification.

| Lot | Part | Prod. date | Shift | Qty OK produced | Shipped (delivery notes) | Arga stock | Action | Result |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| L26070-AR1008-01 | AR-1008 | 2026-03-11 | night | 983 | 983 (AL-26-00154) | 0 | Sorting at customer | 98 NOK |
| L26077-AR1008-01 | AR-1008 | 2026-03-18 | morning | 2,486 | 2,486 (AL-26-00170) | 0 | Sorting at customer | 0 NOK |
| L26084-AR1008-01 | AR-1008 | 2026-03-25 | afternoon | 1,668 | 1,668 (AL-26-00184) | 0 | Sorting at customer | 0 NOK |
| L26091-AR1008-01 | AR-1008 | 2026-04-01 | night | 1,087 | 1,087 (AL-26-00199) | 0 | Sorting at customer | 0 NOK |
| L26098-AR1008-01 | AR-1008 | 2026-04-08 | morning | 1,774 | 1,774 (AL-26-00217) | 0 | Sorting at customer | 0 NOK |
| L26105-AR1008-01 | AR-1008 | 2026-04-15 | afternoon | 2,488 | 2,488 (AL-26-00231) | 0 | Sorting at customer | 0 NOK |

Arga stock (0 parts) blocked on 2026-04-21; 100 % check on checking fixture CF-AR1008.

OEM Norte stock sorted on 2026-04-22–2026-04-23.

# D4 · Root cause analysis

Analysis on 2026-04-25.

Ishikawa diagram (6M)

| Category | Potential cause | Verification / status |
| --- | --- | --- |
| Method | Die change on the night shift without first-off approval (no quality technician on nights) | CONFIRMED |
| Machine | Die locating: two stop blocks on PR-250 bolster, no poka-yoke | CONFIRMED |
| Man | Die setter from L3 covering the night shift | Contributing |
| Material | Coil S-ULTZ-260123 (DC04) | Ruled out |
| Measurement | Checking fixture control at 06:00 found the deviation but the lot had been partially packed | CONFIRMED (non-detection) |
| Environment | — | Not related |

5 Whys — occurrence

| Why |  |
| --- | --- |
| 1 | Why hole position shifted? — Die MT-08 set against the wrong stop block. |
| 2 | Why? — Two stop blocks exist on the bolster and no locating pin prevents the wrong position. |
| 3 | Why was it not detected at setup? — First-off approval by quality is not available on the night shift and was skipped. |

5 Whys — non-detection

| Why |  |
| --- | --- |
| 1 | Why not detected earlier? — Checking fixture control once per shift; parts packed before the 06:00 check were not segregated. |

Root cause — occurrence: Die mis-location on PR-250 during a night-shift die change, without poka-yoke and without first-off approval.

Root cause — non-detection: First-off approval skipped on nights; low checking fixture frequency.

Root cause verification: Reproduced on 2026-04-26: die on the second stop block → +0.8 mm in Y.

# D5 · Permanent corrective actions (chosen)

| Action | Type | Owner | Due date |
| --- | --- | --- | --- |
| Locating pins on the PR-250 bolster for MT-08 (poka-yoke) | Corrective – occurrence | Unai Ezkurra | 2026-05-12 |
| Night-shift first-off approval by the shift leader with checking fixture + photo in the setup log | Corrective – occurrence | Josu Elizalde | 2026-04-28 |
| Checking fixture control every 2 h; packing only after first-off approval | Corrective – detection | Mikel Aranburu | 2026-04-28 |

# D6 · Implementation and validation

Implemented by 2026-05-12. IT-L1-PR250-04 updated to v3.

Validation results

Hole position within ±0.12 mm for 8 weeks.

No recurrence at OEM Norte.

# D7 · Actions to prevent recurrence

| Action | Document / system | Owner | Due date | Status |
| --- | --- | --- | --- | --- |
| Die setup instruction: night-shift first-off approval | IT-L1-PR250-04 v3 | Unai Ezkurra | 2026-05-12 | Done |
| Read-across: locating pins for all dies on PR-250 | Toolroom | Unai Ezkurra | 2026-06-20 | Done |

# D8 · Closure and team recognition

8D closed on 2026-06-28; accepted by OEM Norte.

| Role | Name | Date | Signature |
| --- | --- | --- | --- |
| Customer Quality Engineer (8D leader) | Maialen Irigoyen | 2026-06-28 | signed |
| Quality Manager (champion) | Iñaki Etxeberria | 2026-06-28 | signed |
| OEM Norte SQE (acceptance) | M. Ruiz | 2026-07-01 | accepted |